Lab-2

Implementation of Error Detection Techniques.

THEORY:

Error Detection Codes : The binary information is transferred from one location to another location through some communication medium. The external noise can change bits from 1 to 0 or 0 to 1.This changes in values changes the meaning of actual message and is called error. For efficient data transfer, there should be an error detection and correction codes. An error detection code is a binary code that detects digital errors during transmission. To detect error in the received message, we add some extra bits to the actual data. Without addition of redundant bits, it is not possible to detect errors in the received message. There are 3 ways in which we can detect errors in the received message :

- Parity Bit

- CheckSum

- Cyclic Redundancy Check (CRC)

Parity Bit Method : A parity bit is an extra bit included in binary message to make total number of 1’s either odd or even. Parity word denotes number of 1’s in a binary string. There are two parity system – even and odd parity checks.

- Even Parity Check: Total number of 1’s in the given data bit should be even. So if the total number of 1’s in the data bit is odd then a single 1 will be appended to make total number of 1’s even else 0 will be appended(if total number of 1’s are already even). Hence, if any error occurs, the parity check circuit will detect it at the receiver’s end.

- Odd Parity Check: In odd parity system, if the total number of 1’s in the given binary string (or data bits) are even then 1 is appended to make the total count of 1’s as odd else 0 is appended. The receiver knows that whether sender is an odd parity generator or even parity generator. Suppose if sender is an odd parity generator then there must be an odd number of 1’s in received binary string. If an error occurs to a single bit that is either bit is changed to 1 to 0 or 0 to 1, received binary bit will have an even number of 1’s which will indicate an error.

PROGRAM:

/************EVEN PARITY*************/

clc;

clear;

%**********EVEN PARITY CHECK WITH USER INPUT*************%

% Prompt user to enter a binary bit sequence

x = input('Enter the bit sequence to test for even parity (e.g., 110010): ', 's');

% Initialize counter for number of 1s

t = 0;

% Loop through each bit in the input sequence

for i = 1:length(x)

if(x(i) == '1')

t = t + 1; % Increment by one if a bit is '1'

end

end

% Check if the total number of '1's is not even, then add a bit to make it even

if(mod(t, 2) ~= 0)

y = [x, '1'];

disp('Parity bit generated: 1');

else

y = [x, '0'];

disp('Parity bit generated: 0');

end

% Display input and output sequences

disp('Input bit sequence:');

disp(x);

disp('Bit sequence with even parity bit added:');

disp(y);

output:

/************ODD PARITY*************/

clc;

clear;

%**********ODD PARITY CHECK WITH USER INPUT*************%

% Prompt user to enter a binary bit sequence

x = input('Enter the bit sequence to test for odd parity (e.g., 110010): ', 's');

% Initialize counter for number of 1s

t = 0;

% Loop through each bit in the input sequence

for i = 1:length(x)

if(x(i) == '1')

t = t + 1; % Increment by one if a bit is '1'

end

end

% Check if the total number of '1's is even (i.e., parity not odd), then add a bit to make it odd

if(mod(t, 2) == 0)

y = [x, '1'];

disp('Parity bit generated: 1');

else

y = [x, '0'];

disp('Parity bit generated: 0');

end

% Display input and output sequences

disp('Input bit sequence:');

disp(x);

disp('Bit sequence with odd parity bit added:');

disp(y);

Output:

CheckSum error detection techniques:

In checksum error detection technique, the data is divided into K-segments each of m-bits.

- In the sender’s end the segment is added using 1’s complement arithmetic to get the sum. The sum is complemented to get the checksum.

- The checksum segment is sent along with data segments.

- At the Receiver’s end all the received signals are added using 1’s complement arithmetic to get the sum. The sum is complemented.

- If the result is zero , the received data is accepted ,otherwise discarded.

Example:

Original message =1010100100111001

K=2

M=8

At the sender side

10101001

00111001

11100010 (sum)

00011101(checksum-1’s complement of sum)

At the receiver side

11100010

00011101

11111111(sum)

00000000(checksum 1’s complement)