Program

clc; clear; close all;

%% Parameters

fs = 1000;               % Sampling frequency (Hz)

t = 0:1/fs:1;            % Time vector (1 second duration) f = 5;                   % Signal frequency (Hz)

x = 1.2 * sin(2 * pi * f * t);  % Analog signal (amplitude scaled)

L = 16;                  % Number of quantization levels (4 bits) bits = log2(L);          % Bits per sample

xmax = max(abs(x));      % Maximum amplitude for normalization

%% Quantization

x_norm = (x + xmax) / (2 * xmax);  % Normalize between 0 and 1 xq = round(x_norm * (L - 1));      % Quantize to L levels

%% Binary Encoding (PCM Coding)

pcm = dec2bin(xq, bits);  % Convert to binary strings

%% Display a few samples

disp('First 10 samples of quantized PCM codes:'); disp(pcm(1:10, :));

%% Plot 1: Original Analog Signal & Quantized Signal (Same Figure, Separate Subplots) figure; subplot(2, 1, 1); plot(t, x, 'b');

title('Original Analog Signal-Samin Thapa'); xlabel('Time (s)'); ylabel('Amplitude'); grid on;  subplot(2, 1, 2); stairs(t, xq, 'r');

title('Quantized Signal (PCM Levels)-Samin Thapa'); xlabel('Time (s)'); ylabel('Quantized Level'); grid on;

%% Plot 2: Normalized Signal (Separate Figure) figure;

plot(t, x_norm, 'm'); title('Normalized Signal-Samin Thapa'); xlabel('Time (s)'); ylabel('Normalized Amplitude'); grid on;

%% Reconstruction (Demodulation)

xq_recon = (xq / (L - 1)) * (2 * xmax) - xmax;  % De-normalize back

%% Plot 3: Original & Reconstructed Signal (Same Figure, Separate Subplots) figure; subplot(2, 1, 1); plot(t, x, 'b');

title('Original Analog Signal-Samin Thapa'); xlabel('Time (s)'); ylabel('Amplitude'); grid on;  subplot(2, 1, 2); plot(t, xq_recon, 'r');

title('Reconstructed Signal from PCM'); xlabel('Time (s)'); ylabel('Amplitude'); grid on;

output