Program:

clc;

close all;

% Input: Generator matrix for Hamming(7,4) in standard (4x7) form

g = input('Enter the generator matrix G (4x7): ');

disp('Generator matrix G:');

disp(g);

% Determine the order of the linear code (n=7, k=4 for Hamming(7,4))

[k, n] = size(g);

disp(['The order of the linear code is: n = ' num2str(n) ', k = ' num2str(k)]);

% Generate all possible 4-bit input messages

u = zeros(2^k, k);

for i = 0:(2^k - 1)

binStr = dec2bin(i, k);

u(i+1, :) = binStr - '0';

end

disp('All possible 4-bit messages (u):');

disp(u);

% Generate all codewords using G

c = mod(u * g, 2);

disp('All possible codewords (u * G mod 2):');disp(c);

% Calculate minimum Hamming distance (d_min)

d_min = min(sum(c(2:end, :) ~= 0, 2));

disp(['Minimum Hamming distance (d_min): ' num2str(d_min)]);

% Input: Received 7-bit codeword

r = input('Enter the received 7-bit codeword as a row vector [1 0 1 1 0 1 0]: ');

% Parity matrix P is the rightmost part of G (non-identity part)

P = g(:, k+1:end);

% Construct parity-check matrix H = [P^T | I]

H = [P.' eye(n - k)];

Ht = H'; % transpose for syndrome calculation

% Syndrome calculation: s = r * H^T mod 2

s = mod(r * Ht, 2);

disp('Syndrome of the received codeword:');

disp(s);

% Error detection and correction

error_detected = false;

error_position = 0;

% Check if syndrome matches any column in H

for i = 1:n

if isequal(H(:, i)', s)

r(i) = 1 - r(i); % Correct the error by flipping the bit

error_position = i;

error_detected = true;

break;

end

end

% Display results

if error_detected

disp(['Error detected and corrected at bit position: ' num2str(error_position)]);

else

disp('No error detected.');

end

disp('Corrected codeword:');

disp(r);

Output:

Enter the generator matrix G (4x7): [1 0 0 0 1 1 0;

0 1 0 0 1 0 1;

0 0 1 0 0 1 1;

0 0 0 1 1 1 1]

Generator matrix G:
     1     0     0     0     1     1     0
     0     1     0     0     1     0     1
     0     0     1     0     0     1     1
     0     0     0     1     1     1     1

The order of the linear code is: n = 7, k = 4
All possible 4-bit messages (u):
     0     0     0     0
     0     0     0     1
     0     0     1     0
     0     0     1     1
     0     1     0     0
     0     1     0     1
     0     1     1     0
     0     1     1     1
     1     0     0     0
     1     0     0     1
     1     0     1     0
     1     0     1     1
     1     1     0     0
     1     1     0     1
     1     1     1     0
     1     1     1     1

All possible codewords (u * G mod 2):
     0     0     0     0     0     0     0
     0     0     0     1     1     1     1
     0     0     1     0     0     1     1
     0     0     1     1     1     0     0
     0     1     0     0     1     0     1
     0     1     0     1     0     1     0
     0     1     1     0     1     1     0
     0     1     1     1     0     0     1
     1     0     0     0     1     1     0
     1     0     0     1     0     0     1
     1     0     1     0     1     0     1
     1     0     1     1     0     1     0
     1     1     0     0     0     1     1
     1     1     0     1     1     0     0
     1     1     1     0     0     0     0
     1     1     1     1     1     1     1

Minimum Hamming distance (d_min): 3
Enter the received 7-bit codeword as a row vector [1 0 1 1 0 1 0]: [1 0 1 1 0 1 0]

Syndrome of the received codeword:
     0     0     0

No error detected.
Corrected codeword:
     1     0     1     1     0     1     0