### LAB – 1

### 8-BIT ADDITION

OBJECTIVE: To perform 8-bit Addition of two hexadecimal numbers using accumulator in 8085

PROGRAM:

| Program | Explanation |
| --- | --- |
| MVI A, 05H | Load immediate data 05 into the accumulator |
| MVI B, 04H | Load immediate data 05 register B |
| ADD B | Add data in B to accumulator & store in the accumulator |
| STA C300 | Store the (Result) contents of accumulator to address C300 |
| HLT (RST 1) | End of Program |

MNEMONICS:

| ADDRESS | INSTRUCTIONS | OPCODE |
| --- | --- | --- |
| C000 | MVI A, 05 | 3E |
| C001 |  | 05 |
| C002 | MVI B, 04 | 06 |
| C003 |  | 04 |
| C004 | ADD B | 80 |
| C005 | STA C300 | 32 |
| C006 |  | 00 |
| C007 |  | C3 |
| C008 | RST 1 | CF |

CALCULATION:

INPUT 1 = 05	= 0000 0101

INPUT 2 = 04 = 0000 0100

+   09 = 0000 1000

INPUT 1 = 05	= 0000 0101

INPUT 2 = 02 = 0000 0010

+   07 = 0000 0111

DISCUSSION AND CONCLUSION:

- Instruction set used (Data transfer, Arithmetic, Logical, Branch, Program Control)

In this lab, we wrote an ALP language for 8085 to perform addition of two 8 bit numbers. There are data transfer instructions like MVI, STA. There is an arithmetic instruction ADD.

- Instruction set used based on size (1 byte, 2 bytes, 3 bytes) and

We have 1 byte , 2 bytes and 3 bytes instructions. One byte instructions are ADD B, HLT. Two bytes instruction are MVI A and MVI B and one three byte instruction STA.

- Addressing mode used (Direct, Indirect, Implicit, Immediate)

Adressing mode used are immediate (MVI), direct addressing mode(STA).

- Hence, the program for adding two 8 – bit numbers using accumulator and register has been performed with different sets of data.

### LAB – 2

### 8 BIT MULTIPLICATION

OBJECTIVE: To multiply two 8-bit hexadecimal numbers using 8085 microprocessor

### PROGRAM:

| Address/Label | Program | Explanation |
| --- | --- | --- |
|  | LXI H, C201 | Setting pointer for data |
|  | XRA A | Set Accumulator to '0' to account for product |
|  | MOV B,M | Move Data 1 to Register B from Memory |
|  | INX H | Increment the memory pointer |
|  | MOV D,M | Move Data 2 to Register D from Memory |
| Repeat | ADD D | Add contents of Register D to Accumulator |
|  | DCR B | Decrement Register B content by 1 |
|  | JNZ Repeat | If B is not Zero, got back to Repeat |
|  | INX H | Increment the memory pointer |
|  | MOV M,A | Store Accumulator content (Product) to memory |
|  | INX H | Increment the memory pointer |
|  | HLT | End of program |

### MNEMONICS:

| ADDRESS | INSTRUCTIONS | OPCODE |
| --- | --- | --- |
| C000 | LXI H, C201 | 21 |
| C001 |  | 01 |
| C002 |  | C2 |
| C003 | XRA A | AF |
| C004 | MOV B,M | 46 |
| C005 | INX H | 23 |
| C006 | MOV D,M | 56 |
| C007 | ADD D | 82 |
| C008 | DCR B | 05 |
| C009 | JNZ Repeat | C2 |
| C00A |  | 07 |
| C00B |  | C0 |
| C00F | INX H | 23 |
| C010 | MOV M, A | 77 |
| C011 | INX H | 23 |
| C012 | RST 1 | CF |

INPUT & OUTPUT:

| C201 | INPUT 1 | 5 |
| --- | --- | --- |
| C202 | INPUT 2 | 3 |
| C203 | PRODUCT | F |

CALCULATION:

| 05 | = 0000 0101 |
| --- | --- |
| + 05 | = 0000 0101 |
|  | = 0000 1010 |
| + 05 | = 0000 0101 |
| = 0F | = 0000 1111 |

= 0F H

RESULT: The program to multiply two 8-bit hexadecimal numbers using 8085 was executed.

DISCUSSION AND CONCLUSION:

- Instruction set used (Data transfer, Arithmetic, Logical, Branch, Program Control)

- Instruction set used based on size (1 byte, 2 bytes, 3 bytes) and

- Addressing mode used (Direct, Indirect, Implicit, Immediate)

- Hence, the program for adding two 8 – bit numbers using accumulator and register has been performed with different sets of data.

Hints for Division:

Compare (register or memory) with accumulator (CMP R/M) –

If A less than (R/M), the CY flag is set and zero flag is reset.

If A equals to (R/M), the Zero flag is set and CY flag is reset.

If A greater than (R/M), the CY and Zero flag are reset.

LAB – 3

### 8-BIT DIVISION

OBJECTIVE: To divide two 8-bit hexadecimal numbers using 8085 microprocessor

PROGRAM:

| Address/ Label | Program | Explanation |
| --- | --- | --- |
|  | LDA C200 | Load contents of Accumulator with Divisor from memory |
|  | MOV B,A | Move contents of Accumulator to Register B |
|  | LDA C201 | Load contents of Accumulator with Dividend from memory |
|  | MVI C, 00 | Clear register C to account for Quotient |
| Again | CMP B | Compare Register B with Accumulator |
|  | JC Store | If carry==1, jump to Store |
|  | SUB B | If carry==0, subtract Register B from Accumulator |
|  | INR C | Increment register C by 1 |
|  | JMP Again | Jump to Again , unconditionally |
| Store | STA C203 | Store Accumulator content (Reminder) to |
|  |  | Memory |
|  | MOV A,C | Move contents of Register C to Accumulator |
|  | STA C202 | Store Accumulator content (Quotient) to Memory |
|  | HLT | End of Program |

MNEMONICS:

| ADDRESS | INSTRUCTIONS | OPCODE |
| --- | --- | --- |
| C000 | LDA C201 | 3A |
|  |  | 01 |
|  |  | C2 |
| C003 | MOV B,A | 47 |
| C004 | LDA C200 | 3A |
|  |  | 00 |
|  |  | C2 |
| C007 | MVI C, 00 | 0E |
|  |  | 00 |
| C009 | CMP B | B8 |
| C00A | JC Store | DA |
|  |  | 12 |
|  |  | C0 |
| C00D | SUB B | 90 |
| C00E | INR C | 0C |
| C00F | JMP Again | C3 |
|  |  | 09 |
|  |  | C0 |
| C012 | STA C203 | 32 |
|  |  | 03 |
|  |  | C2 |
| C015 | MOV A,C | 79 |
| C016 | STA C202 | 32 |
|  |  | 02 |
|  |  | C2 |
| C019 | RST 1 | CF |

INPUT & OUTPUT:

i)

| C200 | DIVISOR | 3 |
| --- | --- | --- |
| C201 | DIVIDEND | 6 |
| C202 | REMINDER | 0 |
| C203 | QUOTIENT | 2 |

ii)

| C200 | DIVISOR | 5 |
| --- | --- | --- |
| C201 | DIVIDEND | 9 |
| C300 | REMINDER | 4 |
| C301 | QUOTIENT | 1 |

CALCULATION:

i) Dividend  A=06 ; Divisor B=03

A – B = 06 -03 = 03    : While A>B,  Carry = 0, C = C+1

A – B= 03 – 03=00      : While A = B, Carry = 0, C=C+1

Reminder = 0 , Carry=02, This is our Quotient.

RESULT: Thus program for dividing two hexadecimal numbers using

8085 Microprocessor has been executed

DISCUSSION AND CONCLUSION:

- Instruction set used (Data transfer, Arithmetic, Logical, Branch, Program Control )

- Instruction set used based on size (1 byte, 2 bytes, 3 bytes) and

- Addressing mode used (Direct, Indirect, Implicit, Immediate)

- Hence, the program for adding two 8 – bit numbers using accumulator and register has been performed with different sets of data.

### LAB – 4

### ADDITION OF ‘n’ NUMBERS IN ARRAY

OBJECTIVE: To add ‘n’ numbers in array and find the sum of those numbers using 8085 Microprocessor with memory pointer.

PROGRAM:

| Address | Program | Explanation |
| --- | --- | --- |
|  | LXI H, C201 | Setting pointer for data (No. of data in array) |
|  | MOV B,M | Move number of data to Register B from Memory |
|  | MVI C, 00 | Clear register C to account for Carry |
|  | XRA A | Set Accumulator to '0' to account for product |
| Repeat | INX H | Increment the memory pointer |
|  | ADD M | Add Contents of Memory to Accumulator & store in Accumulator |
|  | JNC Ahead | If carry==0 , go Ahead |
|  | INR C | Increment register C by 1, If carry ==1 |
| Ahead | DCR B | Decrement Register B content by 1 |
|  | JNZ Repeat | If B is not Zero, got back to Repeat |
|  | STA C300 | Store Accumulator content (Sum) to Memory |
|  | MOV A,C | Move contents of Register C to Accumulator |
|  | STA C301 | Store Accumulator content (Carry) to Memory |
|  | HLT (RST 1) | End of Program |

MNEMONICS:

| ADDRESS | INSTRUCTIONS | OPCODE |
| --- | --- | --- |
| C000 | LXI H, C201 | 21 |
|  |  | 01 |
|  |  | C2 |
| C003 | MOV B,M | 46 |
| C004 | MVI C, 00 | 0E |
|  |  | 00 |
| C006 | XRA A | AF |
| C007 | INX H | 23 |
| C008 | ADD M | 86 |
| C009 | JNC Ahead | D2 |
|  |  | 0D |
|  |  | C0 |
| C00C | INR C | 0C |
| C00D | DCR B | 05 |
| C00E | JNZ Repeat | C2 |
|  |  | 07 |
|  |  | C0 |
| C011 | STA C300 | 32 |
|  |  | 00 |
|  |  | C3 |
| C014 | MOV A,C | 79 |
| C015 | STA C301 | 32 |
|  |  | 01 |
|  |  | C3 |
| C018 | RST 1 | CF |

INPUT & OUTPUT:

| C200 | No. of Elements | 6 |
| --- | --- | --- |
| C201 | DATA 1 | 2 |
| C202 | DATA 2 | 4 |
| C203 | DATA 3 | 6 |
| C204 | DATA 4 | 8 |
| C205 | DATA 5 | 0A |
| C206 | DATA 6 | 0C |
| C300 | TOTAL | 2A |
| C301 | CARRY | 0 |

CALCULATION:

| DATA 1 : | 02 | - 0000 0010 |
| --- | --- | --- |
| DATA 2 : | 04 | - 0000 0100 |
| + | 06 | - 0000 0110 |
| DATA 3 : | 06 | - 0000 0110 |
| + | 0C | - 0000 1100 |
| DATA 4 : | 08 | - 0000 1000 |
| + | 14 | - 0001 0100 |
| DATA 5 : | 0A | - 0000 1010 |
| + | 1E | - 0001 1110 |
| DATA 6 : | 0C | - 0000 1100 |
| TOTAL : | 2A | - 0010 1010 |
| CARRY : | 00 |  |

RESULT: Program to add ‘n’ numbers in array and find the sum of those numbers using 8085 Microprocessor with memory pointer was executed.

DISCUSSION AND CONCLUSION:

- Instruction set used (Data transfer, Arithmetic, Logical, Branch, Program Control )

- Instruction set used based on size (1 byte, 2 bytes, 3 bytes) and

- Addressing mode used (Direct, Indirect, Implicit, Immediate)

- Hence, the program for adding two 8 – bit numbers using accumulator and register has been performed with different sets of data.

### LAB – 5

### LARGEST IN ARRAY

AIM: To find the Largest element in an array of size ‘n’ using 8085 Microprocessor.

PROGRAM:

| Address | Program | Explanation |  |
| --- | --- | --- | --- |
|  | LXI H, C200 | Setting pointer for data |  |
|  | MOV B,M | Move number of data to Register B from Memory |  |
|  | INX H | Increment the memory pointer |  |
|  | MOV A,M | Move contents of memory to Accumulator |  |
|  | DCR B | Decrement Register B content by 1 |  |
| Loop | INX H | Increment the memory pointer |  |
|  | CMP M | Compare contents of memory with Accumulator |  |
|  | JNC Ahead | If carry==0, go Ahead i.e., number is smaller |  |
|  | MOV A,M | Move contents of memory to Accumulator |  |
| Ahead | DCR B | Decrement Register B content by 1 |  |
|  | JNZ Loop |  |
|  | STA C300 | If Register B is not equal to '0', go to Loop |  |
|  | HLT (RST 1) | Store Accumulator content (largest number) to Memory |  |

MNEMONICS:

| ADDRESS | INSTRUCTIONS | OPCODE |
| --- | --- | --- |
| C000 | LXI H, C200 | 21 |
|  |  | 00 |
|  |  | C2 |
| C003 | MOV B,M | 46 |
| C004 | INX H | 23 |
| C005 | MOV A,M | 7E |
| C006 | DCR B | 5 |
| C007 | INX H | 23 |
| C008 | CMP M | BE |
| C009 | JNC Ahead | D2 |
|  |  | 0D |
|  |  | C0 |
| C00C | MOV A,M | 7E |
| C00D | DCR B | 5 |
| C00E | JNZ Loop | C2 |
| C011 | STA C300 | 32 |
|  |  | 00 |
|  |  | C3 |
| C014 | RST 1 | 76 |

INPUT & OUTPUT:

| C200 | No. of Elements | 6 |
| --- | --- | --- |
| C201 | DATA 1 | 04 |
| C202 | DATA 2 | 03 |
| C203 | DATA 3 | 01 |
| C204 | DATA 4 | 22 |
| C205 | DATA 5 | 05 |
| C206 | DATA 6 | 0C |
| C300 | TOTAL | 01 |

CALCULATION:

No. of Data =B =6

A=04  	; B=5

M=03 , Compare M with A: Carry=1 B=4

M=01, Compare M with A: Carry=1 ; B=3

M=22, Compare M with A: Carry=0,

A=22  ;B=2

M=05, Compare M with A: Carry=1, B=1

M=0C, Compare M with A: Carry=1, B=0

Hence, A=22  Largest number

DISCUSSION AND CONCLUSION:

- Instruction set used (Data transfer, Arithmetic, Logical, Branch, Program Control )

- Instruction set used based on size (1 byte, 2 bytes, 3 bytes) and

- Addressing mode used (Direct, Indirect, Implicit, Immediate)

- Hence, the program for adding two 8 – bit numbers using accumulator and register has been performed with different sets of data.

### LAB – 6 (8086)

### MULTIPLICATION OF TWO 16-BIT NUMBERS

OBJECTIVE: To multiply 2 numbers (16-bit data) where starting address is 2000 and the numbers are at 3000 and 3002 memory address and store result into 3004 and 3006 memory address using 8086 Microprocessor.

Example –

Algorithm –

- First load the data into AX(accumulator) from memory 3000

- Load the data into BX register from memory 3002

- Multiply BX with Accumulator AX

- Move data from AX(accumulator) to memory

- Move data from DX to AX

- Move data from AX(accumulator) to memory

- Stop

Program –

| Memory | Mnemonics | Operands | Comment |
| --- | --- | --- | --- |
| 2000 | MOV | AX, [3000] | [AX] <- [3000] |
| 2004 | MOV | BX, [3002] | [BX] <- [3002] |
| 2008 | MUL | BX | [AX] <- [AX] * [BX] |
| 200A | MOV | [3004], AX | [3004] <- AX |
| 200E | MOV | AX, DX | [AX] <- [DX] |
| 2010 | MOV | [3006], AX | [3006] <- AX |
| 2014 | HLT |  | Stop |

Explanation –

- MOV is used to load and store data.

- MUL is used to multiply two 16-bit numbers.

- HLT is used to stop the program.

- AX is an accumulator which is used to store the result.

- BX, DX are general purpose registers where BX is used for multiplication and DX is used for result.

INPUT & OUTPUT:

Input 1 = dW = 16 bit number = 0403

Input 2 = dW = 16 bit number = 0708

Result = 001C 3518

| 3000 | Input 1 | 03 |
| --- | --- | --- |
| 3001 |  | 04 |
| 3002 | Input 2 | 08 |
| 3003 |  | 07 |
| 3004 | Result was in AX | 18 |
| 3005 |  | 35 |
| 3006 | Result was in DX | 1C |
| 3007 |  | 00 |

### LAB – 7 (8086)

DISPLAY A STRING ‘Electrical and Electronics Engineering’ using 8086.

Title Display ‘Electrical and Electronics Engineering’

Dosseg

.model small

.stack 100h

.data
String1 db ‘Electrical and Electronics Engineering’, $

.code

Main proc

MOV AX, @data

MOV DS, AX

MOV AH, 09H

MOV DX, offset String1

INT 21H

MOV AH, 4CH

INT 21H

Main endp

End Main

DISCUSSION AND CONCLUSION:

A string is declared in the .DATA section. The name of the string is set to

aString and the type of the data is byte (DB=data byte). Although it is not only a single byte, assemblers are only concerned with the data type of the first item, which is then assumed for all other declarations before a new label. At the end of the string is a 13 and 10 - these are the carriage return and linefeed characters that are used to go to the next line. The last '$' is needed by the output function to signal the end of the string.

Before the program can use the data in the data segment, the DS register must first be set up appropriately. Unlike the CS register which is always set when the program starts, the DS register must be explicitly set to point to the DATA segment. The first "mov" command gets the SEGment of the DATA segment and stores it in the AX register. The second "mov" command sets the DS register value from the AX register. The reason why two commands are necessary is because the 8086 CPU does not have a command to directly set a value into a segment register.

The value of 9 is inserted into the AH register to select sub-function 9 of the interrupt 21h DOS interrupts. This interrupt requires that the DS:DX segment:offset pair point to the string to be output. In this case, DS already points to the segment containing the string. So we just set the DX register to the OFFSET of the string.

Interrupt 21h is called to output the string and the program terminates like before.