In 8086 Assembly, both Procedures and Macros are used to make code modular and reusable, but they work in fundamentally different ways. Think of a Procedure as a "shared tool" everyone takes turns using, while a Macro is a "template" that gets copied and pasted everywhere you need it.

1. Procedures (Near/Far)

A procedure is a subprogram that is stored in one place in memory. When you call it, the CPU jumps to that memory address, executes the code, and then returns to where it left off.

Example: A procedure to add two numbers

Code snippet

; Define the Procedure

ADD_NUMS PROC

ADD AX, BX

RET           ; Returns to the instruction after 'CALL'

ADD_NUMS ENDP

; Calling the Procedure

MOV AX, 5

MOV BX, 10

CALL ADD_NUMS     ; Execution jumps to ADD_NUMS

2. Macros

A macro is a "find and replace" instruction for the Assembler. When you use a macro, the Assembler takes the actual code of the macro and inserts it directly into your program wherever you invoked it.

Example: A macro to print a character

Code snippet

; Define the Macro

PRINT_CHAR MACRO CHAR

MOV AH, 02H

MOV DL, CHAR

INT 21H

ENDM

; Using the Macro

PRINT_CHAR 'A'    ; Assembler replaces this with the 3 lines of code above

PRINT_CHAR 'B'    ; Replaces this too, adding 3 more lines to memory

Key Differences

| Feature | Procedure | Macro |
| --- | --- | --- |
| Expansion | Called during runtime. | Expanded during assembly time. |
| Memory | Saves space; code exists only once. | Uses more space; code is duplicated. |
| Speed | Slower (overhead of calling/returning). | Faster (no jumping, just execution). |
| Parameters | Passed via registers or stack. | Passed as arguments during expansion. |
| End Command | Ends with RET. | Ends with ENDM. |

Which one should you use?

- Use Macros for very short snippets of code (1-5 lines) where speed is critical.

- Use Procedures for complex logic or long sequences of code to keep your file size small.