Structure of Memory size of 8085

Detailed structure of 64KB memory with data, address and control lines

Block diagram for RAM and ROM

Interfacing a 8085 microprocessor with 64KB RAM

- Design a memory circuit to interface a 2 K-byte RAM to an 8085 microprocessor with starting address 7000H.

Step 1

Find the number of address lines and chip select lines

Represent them in two form 2N x byte, where N represents the number of address lines

2K byte = (2 x 1024 ) x byte

= 21 x 210 x byte

= 211 x byte

which means 11 addr lines (A0 to A10) required to go to the memory

But microprocessor has 16 addr lines, so remaining 5 addr lines(A11 to A15 ) selects the chip

Therefore, address lines = 11 (A0 to A10) and

Chip select lines = 5 (A11 to A15)

Step 2

Check if the starting address is given or not.

Case 1: Starting address given

If given use that as the starting address say 7000H

Then, write the range for as single RAM chip as follows

A15   A14   A13    A12   A11    A10	               	                   A0

7000H =  0      1      1      1        0      0     0     0       0   0   0   0      0  0  0  0

77FFH  = 0      1      1      1        0      1     1    1        1   1   1   1     1  1  1  1

Case 2: Starting address not given

If not given assume starting address as 0000H

Then, write the range for a single RAM chip as follows

A15   A14   A13    A12   A11	 A10		                            A0

0000H =  0      0      0      0       0    	0     0    0      0   0  0  0     0  0  0 0

07FFH  = 0      0      0      0       0    	1     1    1      1  1  1  1     1  1  1  1

In both the cases we find one thing remains same that is chip select lines, while address lines change its values.

**Note: These chip select lines should be given to the logic gates in such a way that they output logic 0 to enable chip select lines on the given memory chip.

Step 3

Draw the interfacing figure with the above address lines, chip select lines and necessary control signals.

Fig: Circuit interfacing 2KB RAM with 8085 microprocessor

Fig: Memory Map (Case 1)					Fig: Memory Map (Case 2)

- Design a memory circuit to interface a 32K-byte RAM and 32K-byte ROM to an 8085 microprocessor with starting address 1000H. Find the address range.

Note: Write the address range yourself. Homework.